Kristin Kovach

OSU Histology Core Facility

6/2/2014

Immunohistochemical Staining Procedure for Antibodies:

MLH1, clone ES05 Novacastra, cat.#  NCL-L-MLH1

MSH2, clone FE11, Calbiochem, cat.# NA27

MSH6, clone EP49, Epitomics, cat.# AC-0047

PMS2, clone A16-4, BD Pharmingen, cat.# 556415

Investigators:  Dr. Frankel and Heather Hampel

Paraffin embedded tissue was cut at 4 microns and placed on positively charged slides.  Slides with specimens were then placed in a 60 C oven for 1 hour and cooled. Slides were then placed on the Leica BondMax Autostainer. All slides are deparaffinized and rehydrated on-line with Bond Dewax Solution (product code AR9222) and 100% alcohol.

All slides were stained with the in-house created protocol, OSU60. Slides were quenched for 5 minutes in a 3% hydrogen peroxide solution to block for endogenous peroxidase. Antigen retrieval was performed on-line using Leica’s Bond Epitope Retrieval Solution 1 (ER1, product code AR9961) for MLH-1, MSH-2 and PMS-2 for 20 minutes, and Bond Epitope Retrieval Solution 2 (ER2, product code AR9640) for MSH-6 for 20 minutes. Primary antibodies were incubated for 1 hour at room temperature.  The detection system used for all antibodies was Leica’s Bond Polymer Refine Detection (product code DS9800). Lastly sections were incubated with DAB mixed on-line for 10 minutes. Slides were then counterstained in Richard Allen hematoxylin, dehydrated through graded ethanol solutions and coverslipped.

Incubation times of

Antibody     Antigen Retrieval         Dilution             Primary	            Secondary          Tertiary             DAB

MLH1         ER2 20 min.               1:500	    60 min.          20 min.              20 min.            10 min.        	          .

MSH2          ER2 20 min.	             1:3000	    60 min	          20 min. 	     20 min.           10 min.

MSH6          ER1 20 min.              1:800              60 min.          20 min.              20 min.            10 min.

PMS2           ER2 20 min.              1:300              60 min.          20 min.              20 min.            10 min.